# MicroPython Application — Edge AI Inventory System

This is the on-device MicroPython firmware that runs on the Infineon PSoC6 AI Evaluation Kit. It acquires radar data from the BGT60TR13C sensor, classifies it using a DeepCraft neural network, and reports results to a remote inventory server over Wi-Fi.

## File Structure

micropython_application/
├── boot.py                 # Runs on power-up: Wi-Fi, imports, radar init
├── main.py                 # Detection loop: inference + server reporting
├── config.py               # All device/network configuration (auto-generated)
├── wifi.py                 # Wi-Fi connection manager
├── api_client.py           # HTTP client for the inventory REST API
├── radar_lib.py            # Radar controller + signal processing utilities
│
├── lib/                    # Low-level radar & DSP libraries
│   ├── BGT60TRXX.py        # BGT60TR13C sensor driver (SPI, FIFO, chirp config)
│   ├── BGT60TRXX_define.py # Register map and constants for the BGT60 sensor
│   ├── dft.py              # Optimised DFT implementation (@viper/@native)
│   ├── dftclass.py         # High-level FFT class wrapping dft.py
│   ├── polar.py            # Polar coordinate conversion utilities
│   └── window.py           # Windowing functions (Hann, Blackman-Harris, etc.)
│
├── BG60/                   # Reference/backup copies of radar driver files
│
└── Models/                 # Trained DeepCraft model variants
    ├── 13_02_small/        # Early small model (13 Feb iteration)
    │   └── deepcraft_model.mpy
    ├── Medium_final/       # Medium-sized final model
    │   └── deepcraft_model.mpy
    └── Model_final_small/  # ★ Final small model (currently deployed)
        └── deepcraft_model.mpy

## Boot Sequence

MicroPython executes files in a fixed order: boot.py → main.py.

### boot.py — Hardware Initialisation

Runs once on every power-up or hard reset. Responsible for:

- Garbage collection — gc.collect() to maximise available RAM

- Import heavy modules early — deepcraft_model, wifi, radar_lib (loaded once to avoid repeated allocation)

- Wi-Fi connection — calls wifi.connect(), catches timeouts gracefully

- Radar sensor init — creates a global RadarController instance and calls setup():

  - Configures the SPI bus (50 MHz, MSB-first)

  - Sets chirp parameters (start frequency, bandwidth, ADC divider, 128 samples/chirp)

  - Resets and initialises the BGT60TR13C sensor

radar = radar_lib.RadarController()
radar.setup()

Important: The radar object lives in the __main__ module scope. Later code accesses it via getattr(__main__, 'radar').

### main.py — Detection Loop

Runs immediately after boot.py. Contains:

- Model initialisation — loads the DeepCraft .mpy model, queries input/output dimensions, allocates array.array('f') buffers

- Detection loop — repeats every DETECTION_INTERVAL seconds:

  - Ensures Wi-Fi is connected (auto-reconnect on drop)

  - Calls run_inference() to classify the current radar frame

  - Sends the result to the server via api_client.send_detection()

#### run_inference() — Data Flow

Radar ADC (128 samples) → get_raw_samples() → normalize [0, 1] → enqueue → dequeue → label

| Step | Description |
| --- | --- |
| 1 | radar.get_raw_samples() triggers a chirp, reads the FIFO, unpacks ADC data |
| 2 | Each sample is divided by 4095.0 to normalize from 12-bit range to [0, 1] |
| 3 | model.enqueue(buffer) feeds the frame into the model’s sliding window |
| 4 | model.dequeue(output) returns 0 when the window is full and a prediction is ready |
| 5 | The class with the highest probability is returned as (label, confidence) |

DeepCraft return codes: - 0 — prediction ready (output written to buffer) - -1 — sliding window not yet full, keep feeding - -2 — memory allocation error (fatal)

## Radar Library (radar_lib.py)

### RadarController Class

Encapsulates the BGT60TR13C radar sensor configuration and data acquisition.

#### Constructor Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| start_freq | 57_500_000 | Chirp start frequency (57.5 GHz) |
| bandwidth | 5_000_000 | Chirp bandwidth (5 MHz) |
| adc_div | 60 | ADC clock divider |
| samples_per_chirp | 128 | Number of ADC samples per chirp |
| no_of_chirps | 1 | Number of chirps per frame |

#### SPI Pin Mapping (PSoC6)

| Signal | Pin |
| --- | --- |
| SCK | P12_2 |
| MOSI | P12_0 |
| MISO | P12_1 |
| CS | P12_3 |
| RST | P11_1 |
| IRQ | P11_0 |

#### Key Methods

| Method | Description |
| --- | --- |
| setup() | Resets and initialises the sensor hardware |
| get_raw_samples() | Triggers a frame → reads FIFO → unpacks → returns 128 ADC floats |

#### Utility Functions (Module-Level)

| Function | Description |
| --- | --- |
| compute_fft_1d(x) | Pure-Python recursive Cooley-Tukey FFT (power-of-2 length) |
| compute_frobenius_norm(data) | Frobenius norm of a 1D or 2D array (supports complex values) |

Note: These utility functions are not used in the current raw-data model but are available for FFT-based models.

### lib/ — Low-Level Radar Libraries

| File | Purpose |
| --- | --- |
| BGT60TRXX.py | Full sensor driver: SPI register access, FIFO read, chirp config, frame triggering, data unpacking, signal processing (runHighPassFilter, fillFFT_data, fft_to_dB, runAntiCouplingFilter) |
| BGT60TRXX_define.py | Register addresses and bit masks for the BGT60TR13C |
| dft.py | Performance-optimised DFT using MicroPython @viper and @native decorators |
| dftclass.py | Object-oriented FFT wrapper (DFT.FORWARD, DFT.INVERSE) |
| polar.py | Cartesian-to-polar conversion for FFT results |
| window.py | Windowing functions for spectral analysis |

## DeepCraft Model Variants

Three model variants are stored under Models/. Each contains a single deepcraft_model.mpy file compiled from the DeepCraft cloud platform.

| Model | Directory | Description |
| --- | --- | --- |
| Early Small | 13_02_small/ | Initial small model from 13 Feb. Trained on FFT data (65 features). |
| Medium Final | Medium_final/ | Larger model variant with higher accuracy but more RAM/latency. |
| Final Small ★ | Model_final_small/ | Currently deployed. Trained on raw 128-sample radar vectors. Smallest footprint. |

### DeepCraft API

The compiled .mpy model exposes the following API:

import deepcraft_model

model = deepcraft_model.DEEPCRAFT()
model.init()

input_dim  = model.get_model_input_dim()   # → 128
output_dim = model.get_model_output_dim()   # → 3

model.enqueue(input_buffer)     # Feed one time step (array.array('f'))
result = model.dequeue(output)  # Check for prediction

### Output Labels

The model classifies radar frames into 3 classes:

| Index | Label | Description |
| --- | --- | --- |
| 0 | empty_box | Empty container detected |
| 1 | table | Flat surface / table detected |
| 2 | unlabeled | No clear classification |

## Configuration (config.py)

This file is auto-generated by full_setup.py during device provisioning. All runtime parameters are centralised here.

# ---- Wi-Fi ----
WIFI_SSID = "YourNetwork"
WIFI_PASS = "YourPassword"
WIFI_TIMEOUT = 15               # seconds

# ---- Server ----
SERVER_HOST = "192.168.0.129"
SERVER_PORT = 5000
API_BASE = "http://192.168.0.129:5000"

# ---- Device Identity ----
DEVICE_ID = 4                   # Assigned by server on registration
API_KEY = "abc123..."           # Used in X-API-Key header

# ---- Detection Loop ----
DETECTION_INTERVAL = 10         # seconds between inference cycles

# ---- Hardware ----
LED_PIN = "P13_7"               # On-board LED

## API Client (api_client.py)

Communicates with the inventory server over HTTP using MicroPython’s urequests module. All requests include an X-API-Key header for authentication.

### API Routes

| Method | Endpoint | Description | Body |
| --- | --- | --- | --- |
| GET | /api/health | Health check — returns 200 if server is up | — |
| POST | /api/detections | Submit a detection event | { device_id, label, confidence, status } |
| GET | /api/inventory | Fetch current inventory state | — |

### Functions

#### health_check() → bool

Pings the server’s health endpoint. Returns True if the server responds with HTTP 200.

#### send_detection(label, confidence, status="detected") → dict | None

Posts a classification result to the server.

# Example payload sent:
{
    "device_id": 4,
    "label": "empty_box",
    "confidence": 0.87,
    "status": "detected"
}

Response: returns parsed JSON on 201 Created, or None on failure.

#### get_inventory() → list

Fetches the current inventory from the server. Returns a list of item dicts.

### How HTTP Requests Work on MicroPython

MicroPython uses urequests (a minimal HTTP client) instead of Python’s requests library:

import urequests
import ujson

# GET request
response = urequests.get("http://192.168.0.129:5000/api/health")
print(response.status_code)  # 200
response.close()             # Always close to free the socket!

# POST request with JSON body
headers = {
    "Content-Type": "application/json",
    "X-API-Key": "your-api-key"
}
payload = ujson.dumps({"label": "empty_box", "confidence": 0.9})

response = urequests.post(
    "http://192.168.0.129:5000/api/detections",
    data=payload,
    headers=headers
)
print(response.status_code)  # 201
data = response.json()       # Parse JSON response
response.close()

⚠️ Important: Always call response.close() after reading the response. MicroPython has limited socket resources; unclosed connections will cause memory leaks and eventual crashes.

## Wi-Fi Manager (wifi.py)

Handles all Wi-Fi operations using MicroPython’s network module in STA (station) mode.

| Function | Description |
| --- | --- |
| connect() | Connect to the configured network. Raises RuntimeError on timeout. |
| disconnect() | Deactivate the WLAN interface. |
| is_connected() | Returns True if currently connected. |
| get_ip() | Returns the current IP address string, or None. |
| ensure_connected() | Auto-reconnects if the connection was dropped. Called every cycle. |

## Quick Reference — Full Execution Flow

┌─────────────────────────────────────────────────────────────┐
│                        POWER ON                             │
├─────────────────────────────────────────────────────────────┤
│  boot.py                                                    │
│    ├─ gc.collect()                                          │
│    ├─ import deepcraft_model, wifi, radar_lib               │
│    ├─ wifi.connect()                                        │
│    └─ radar = RadarController() → radar.setup()             │
├─────────────────────────────────────────────────────────────┤
│  main.py                                                    │
│    ├─ model = DEEPCRAFT() → model.init()                    │
│    ├─ Allocate input/output buffers (128 × float32)         │
│    └─ LOOP every 10s:                                       │
│         ├─ wifi.ensure_connected()                          │
│         ├─ run_inference()                                  │
│         │    ├─ radar.get_raw_samples()   → 128 ADC values  │
│         │    ├─ normalize → [0, 1]                          │
│         │    ├─ model.enqueue(buffer)                       │
│         │    └─ model.dequeue(output) → (label, confidence) │
│         ├─ api_client.send_detection(label, confidence)     │
│         └─ time.sleep(10)                                   │
└─────────────────────────────────────────────────────────────┘