# Reproduction Guide: Edge AI Inventory System

This guide explains how to set up and run the complete system, from the Flask backend to the MicroPython edge device.

## Prerequisites

Before starting, ensure you have the following installed: * Python 3.10+ (for Backend and Setup script) * Node.js 18+ (for Next.js Frontend) * mpremote (pip install mpremote) * PSoC6 Device connected via USB and flashed mpy interpreter

## Quick Start (Recommended)

The easiest way to reproduce the project is using the automated setup script.

- Clone the Repository:

- git clone https://github.com/CuzImHarry/micropython-hands-on-course.git
cd micropython-hands-on-course/project

- Run the Setup Script:

- python full_setup.py

  - Step 1: Provide your WLAN SSID and Password when prompted.

  - Step 2: Enter a name and location for your device.

  - Step 3: The script will automatically start the servers, register your device, update the configuration, and flash all files (including libraries) to your PSoC6.

- Access the Dashboard: Open http://localhost:3000 in your browser to see the live inventory and device status.

## Manual Reproduction

If you prefer to set up the components individually:

### 1. Backend & Frontend

cd Serverapp
./setup_and_run.bat

- This will create a virtual environment, install Python/Node dependencies, and launch both servers in separate windows.

### 2. MicroPython Device

- Navigate to micropython_application/.

- Update config.py with your WiFi credentials and the Server IP.

- Register your device via the API (POST /api/devices) to get a DEVICE_ID and API_KEY.

- Flash the files:

- mpremote cp -r lib/ :/
mpremote cp *.py :

## Project Structure

- Serverapp/: Flask API and Next.js Dashboard.

- micropython_application/: MicroPython code and libraries for PSoC6.

- full_setup.py: The main automation and deployment script.

[!NOTE] Ensure your PSoC6 is running a compatible DeepCraft AI model for object classification to see active detection events in the dashboard.